According to new syllabus of all

intermediate boards of Sindh.

Prof. Rawala’s ……..

Physics

Helping book

For

Class XI

(All science students)

Unit -10

D.C circuits.

Notes ……………………………………………………… … (03)

Multiple choice questions M.C.Qs.	………….…......................   .(13)

Numericals from text book and Karachi board papers……………..….(15)

Contents

- Ohm’s law, resistance, Resistor color code.

- Resistivity, Resistance, resistivity and temperature coefficient.

- Conductance and conductivity.

- Electromotive force, internal resistance.

- Power dissipation in resistors.

- Kirchoff’s first and second laws.

- Wheatstone bridge.

- Thermoelectric current.

Thoroughly revised and

updated

2026 – 2027



Detailed notes.

Multiple Choice Questions (M.C.Q’s).

Solution of text book numericals.

MCQs and solution of numericals from

Karachi board past papers.

Prof.(r) Mohammed Hussain Rawala.

B.Sc. (Hons.), M.Sc.(Physics)

Former head of physics department

Saint Patrick’s college, Karachi.

Unit. 10 …..

D.C Circuits

Ohm’s law:

Q.No.1: State and explain Ohm’s law?

Ans:  According to Ohm’s law:

“Potential difference “DV ” across the ends of a conductor is directly

proportional to Current “ I” flowing through it provided temperature

and other physical conditions of the conductor do not change”.

DV  I

			                                      DV = R I

Where “R” is the constant of proportionality, it is known as “Resistance of the conductor”

Its value depends upon dimensions, material and temperature of the conductor.

I

DVI

DV

DV

Hence the product of current “I” and resistance “R” of a

conductor is equal to the potential difference “DV” across its ends, provided temperature and other physical conditions of the conductor do not change. The graph between potential difference “DV” and the resulting current “I” flowing through it, is a straight line if Ohm’s law is obeyed.

A material that obeys Ohm’s law is called Ohmic conductor.

Q.No.2: Give an example in which Ohm’s law is not valid.

Ans: When current is passed through an electric bulb its

temperature rises.

Circuit representation

R

It converts part of electrical energy into heat and a part into light. In this case since the temperature of filament of the bulb rises, therefore, current flowing through it will not be directly proportional to the potential difference across its ends. In other words, Ohm’s law will not be obeyed.

Q.No.3: What is resistance? How is it offered?

Ans: It is the opposition offered by a conductor to the flow of electric current through it

When current is passed through a conductor, free electrons move through the conductor in one direction. These free electrons suffer elastic collisions with the vibrating atoms of the conductor. As a result of which their motion is opposed. This opposition to the flow of current (or to the motion of free charges) is called resistance of the conductor.

Note that, with a few exceptions, resistance of materials increases with a rise in temperature and decreases with a fall of temperature.

- The change in resistance with temperature, above and below 0C, over a wide range of temperature is uniform.

Also note that, resistance of semiconductors decreases with a rise in temperature.

Q.No.4:On what factors does the resistance of a conductor depend?

Ans:  Resistance offered by a conductor depends upon:

- Material of the conductor.

- Geometrical dimensions (length, area of cross section etc.) of the conductor.

- Temperature of the conductor.

Q.No.5: What is the unit of resistance?

Ans: The S.I unit of resistance is “ohm ”.

Resistance of a conductor is said to be one ohm (1) if it allows one ampere (1 A) current to pass through it when a potential difference of one volt (1V) is maintained across it.

1 =  1A .

1V

In electronic circuits resistors are used to control the flow of current within desired values.

Q.No.6: Why does the resistance of a conductor increase with a rise in its temperature? Why is Ohm’s law not obeyed when temperature of the conductor rises?

Ans: When temperature of a conductor is raised, vibrational kinetic energy of its atoms increases. Atoms of the conductor now vibrate more vigorously and through longer distances, as a result of which the chances of collisions between these rapidly vibrating atoms and the free electrons flowing through it increases. Due to the increased number of collisions, the opposition to the movement of free electrons or in other words, resistance of the conductor increases.

Similarly, when temperature of a conductor is lowered, the atoms of the conductor vibrate slowly with decreased amplitude of vibrations, due to which chances of their collisions with free electrons or in other words, resistance of the conductor decreases.

Ohm’s law is valid as long as, along with the other physical properties, mainly temperature of the conductor must not change, because if temperature of the conductor changes it’s resistance will also change. Since resistance “R” is a constant of proportionality in Ohm’s law, therefore, whenever resistance of a conductor changes while current is flowing through it, Ohm’s law will not be obeyed.

In other words, when temperature of the conductor changes, potential difference across the conductor will not be directly proportional to current flowing through it (V  I).

Resistor color code:

In electronic circuits carbon resistors are used for introducing high resistances for various purposes. They are small, their value is indicated by a color code.

Color code is in the form of usually four colored coaxial bands in a particular order. Following table having four columns shows how to use the color code.

First and second bands represent number or significant value, 3rd band represents multiplier whereas the fourth band indicates tolerance (in other words, possible variation in resistance, it is expressed in %).

| Color | Value (1st 2nd band) | Multiplier | Tolerance |
| --- | --- | --- | --- |
| Black | 0 | 100 | Gold ± 5% |
| Brown | 1 | 101 | Silver ± 10% |
| Red | 2 | 102 | No color ± 20% |
| Orange | 3 | 103 |  |
| Yellow | 4 | 104 |  |
| Green | 5 | 105 |  |
| Blue | 6 | 106 |  |
| Violet | 7 | 107 |  |
| Grey | 8 | 108 |  |
| White | 9 | 109 |  |

To understand the color code, let us assume that the first color band is Orange, second is Blue, third is Violet and the fourth is Gold. Substituting 3 (for Orange),  6 (for Blue),107(for Violet) and ± 5% (for Gold) we will get the value of resistance as R=35107 ± 5%.

Resistivity:

Q.No.1:What is resistivity (or specific resistance) of a conductor?

Ans:  Resistivity or specific resistance of a conductor may be defined as:

“The resistance offered by a conductor of

unit length and unit area of cross section”.

OR

“Resistivity is the resistance offered

by unit volume of a conductor”.

Q.No.2: What is the unit of resistivity?

Ans: The unit of resistivity is -m (ohm-meter).

Q.No.3: On what factors does the resistivity of a conductor depend?

Ans: Resistivity of a conductor depends upon its material and upon it’s temperature.

At a given temperature resistivity of a conductor depends only upon it’s material.

- Silver is the best conductor of electricity, because its resistivity is the lowest

among all the conductors. In other words, its unit volume offers minimum resistance to the flow of electric current among all the conductors.

- Some alloys have high resistivity, some of them find important use under

suitable conditions where high resistance is required, e.g. in an electrical room heater usually nichrome is used because it’s resistivity is highest.

Note that, resistivity of semiconductors is negative it means that resistance of semiconductors decreases with a rise in temperature.

Deriation:

At a given temperature resistance of a conductor is directly proportional to its length “L” and inversely proportional to its area of cross section “A”:

R  L		&		R   1 .

A

OR		R   L .               	     R =   L .        ……….. (i)

A   			     A

Where “” is a constant of proportionality, it is known as resistivity of the conductor.

At a constant temperature the value of resistivity “” depends upon material of the conductor. Usually the value of “” for conductors is low and for insulators (or bad conductors) its value is high.

From the above formula:		 =  R A .

L

Resistivity “” may be defined as the resistance offered by a conductor of unit length (L = 1) and unit area of cross section (A = 1). (Or resistance offered by its unit volume).

S.I unit of resistivity “r” is ohm-m (Wm).

Resistance, resistivity and temperature coefficient:

Resistance of a conductor of given material changes with temperature, hence if Ro is its resistance at 0C and RT at TC then:

RT – R0  R0 T

RT – R0 = a R0 T

Where “a” is the constant of proportionality known as temperature coefficient of the given material (“a” may be defined as the change in resistance per unit resistance per degree rise in temperature), its value depends upon material of the conductor. Its unit is (C) –1 or K–1.

RT = R0 + a R0 T

RT = R0 (1+ a T)

Hence resistivity of a conductor of given material increases with temperature. if “0” and “T” are resistivities of a conductor of given material at 0C and TC respectively, then in terms of resistivities the above equation becomes:

T = 0 (1+ aDT)

This equation can be used to find the resistivity of a material at any temperature provided its resistivity at 0C and its temperature coefficient “a” are known (this equation depends purely on material).

 Temperature coefficient of resistance  “a” is positive for most of the conductors, showing that their resistance increases with a rise in their temperature. But in case of  semiconductors value of “a” is negative, showing that their resistance decreases with a rise in their temperature.

In semiconductors number of charge carriers (conduction electrons, holes) increases with a rise in its temperature due to which its “a” is negative. In other words, its resistance decreases.

Conductance and conductivity:

Q.No.5: What are conductance and conductivity of a conductor?

Ans: Conductance of a conductor is the reciprocal of its resistance R, similarly it’s conductivity “s” is the reciprocal of its resistivity “r” (i.e.  s = 1/ r).

Unit of conductance is Siemens “S”.

Unit of conductivity is Siemens/meter “S m–”.

In terms of conductivity “s” equation (i) becomes:

R =  L .       ……….. (ii)

s A

Conductance is a measure of how good or how bad a conductor is. If conductance of a conductor is low, its resistance “R” and resistivity “r” will be high and for a given potential difference it will allow less current to pass through it, in other words, it will be a bad conductor of electricity. But if conductance of a conductor is high, its resistance “R” and resistivity “r” will be low and for a given potential difference it will allow more current to pass through it, in other words, it will be a good conductor of electricity.

Electromotive force emf:

When current is passed through an electrical circuit, movement of charges (free electrons) is opposed (due to resistance of the conductor). To overcome this opposition charges will have to do work. This work done dissipates energy of the charges and appears as heat. Charges will, therefore, continuously lose energy. To maintain a constant current some source of power is needed. This source must be able to supply power equal to the power dissipated. Strength of such a source is measured by its electromotive force emf.

Hence, the electromotive force “E” of a source of current (Cell, battery, electric generator etc.) may be defined as:

The potential difference across its terminals when

no current is drawn from it.

Internal resistance:

Q.No.1: What is internal resistance?

Ans: Internal resistance of a source is:

“The opposition to the flow of charges

through a source of emf”.

Q.No.2: What causes internal resistance in a cell, a battery?

Ans: In the external electrical circuit conventional current flows from positive to negative terminal of a source of emf. To complete the circuit same amount of current also flows through the source from negative to positive terminal. While passing through the source, charges suffer collisions with the atoms of the electrolyte. This opposition to the flow of charges while passing through a source is called internal resistance of the source. Internal resistance is offered by the electrolyte of a cell or a battery but in case of a generator internal resistance is offered by wire of its coil.

Q.No.3: What is terminal potential difference V and what is its relation with emf?

Ans: Terminal potential difference is the potential difference available across the terminals of a source for the external circuit.

Explanation and derivation:

A battery (or any source) may be considered as a source of pure emf “E” and an internal resistance “r” connected in series to it. If “R” is the total resistance of the circuit connected externally across the battery, then according to Ohm’s law current “I” flowing through the circuit (having “r” and “R” connected in series to it) is given by:

I =     E    .

(R + r)

E = I R + I r

“I r” is the potential drop within the source across its internal resistance “r”. Hence the potential difference actually acting across the external resistance “R” is “I R”. It is less than emf “E” of the source. Let “I R” be represented by “V”, then

E = V + I r

Hence,			   V = E – I r

“V” is known as the terminal potential difference. When current is delivered by a source, terminal potential difference “V” will be less than it’s emf “E”, but when no current is drawn there will be no potential drop across the internal resistance, therefore, terminal potential difference “V” will be equal to its emf “E”. Larger the value of current drawn from a source larger will be the potential drop within the source and smaller will be the terminal potential difference “V” available for the external circuit.

Power dissipation in resistors:

When current is passed through a conductor, free electrons flowing through the

conductor suffer elastic collisions with the vibrating atoms. During these collisions a part of electric potential energy of these free electrons is transferred to the atoms, as a result of which kinetic energy of atoms increases, resulting in a rise in temperature of the conductor. Electrical energy is, therefore, converted into heat energy.

Hence power dissipation of any electrical device is the rate at which electrical energy is converted into heat when current passes through it.

In other words, we can say that free electrons have to do work to overcome the

opposition (resistance of the conductor) to their flow. This work done by the electrons is equal to the  amount of heat produced.

If current “I” is passed through the conductor for time “t” under a potential difference “V”

then by definition:

Work done per unit charge = V

Work done to move charge “Q” from one end to the other = QV

But    	                       I =  Q 			Q = It

t

		Work done to move charge “Q” from one end to the other = V I t

Since work done by free electrons to overcome resistance “R” of the conductor is equal to the amount of heat produced, hence:

Heat produced = V I t

But power is the rate of doing work or it is the rate at which one form of energy is

converted into the other (in this case electrical energy is converted into heat energy).

Power = Work or energy		P = V I t

Time			      t

P = V I

But according to Ohm’s law:		V = I R

P = I2 R

P =  V2.

R

S.I unit of power is watt. But watt (w) is a small unit of power, it’s commonly used multiples are kw and Mw:

1 kilowatt     (kw)  = 103 w

1 megawatt (Mw) = 106 w

Old unit of power (which is still used in some cases) is horsepower (hp).

1 horsepower (hp) = 746 w = 0.746 kw.

Q.No.1: What is kilowatt-hour (kwh)?

Ans: Kilowatt-hour is the unit of energy, it is used to measure electrical energy on

commercial scale.

I kilowatt-hour (1kwh) is that amount of energy which is delivered by the electric current by constant power of 1 kilowatt (kw) in 1 hour (h).

Since:		Power =  work (or energy)

time

OR		P =  E 			E = P  t

t

Hence energy can be expressed in terms of power unit.

If power is 1 kilowatt and time taken is 1hour, then:

Energy E = 1 kw 1 h = 1 kwh.

But	 1kw = 103 w	    &	1 h = 3600 s

	1 kwh = 103 w    3600 s

1 kwh = 103 J/s  3600 s

1 kwh = 3.6  106 J

1 kwh = 3.6 MJ.

This formula gives the relation between the commercial unit of energy “kwh” and the standard S.I unit of energy “J”.

1kwh electrical energy is commonly known as a unit of electricity.

Kirchoff’s laws:

Kirchoff’s laws are useful in analyzing complex networks (a complex network is that electrical circuit which has two or more voltage sources along with different series and parallel combinations of resistances).

Kirchoff’s first law:(OR Kirchoff’s Current law KCL)

According to Kirchoff’s first law:

Total current flowing towards a junction is equal to the

total current flowing away from it.

Kirchoff’s first law is also known as the Kirchoff’s current law KCL, it is the direct result of law of conservation of charge.

Current is the rate of flow of charge (amount of charge flowing per second). Since charge can-not be created nor can it be destroyed, therefore, total amount of charge flowing per second towards must be equal to the total amount charge per second flowing away from a junction.

Mathematically we can write:

S Itowards = S Iaway

Kirchoff’s second law: (OR Kirchoff’s Voltage Law KVL)

According to Kirchoff’s second law:

The sum of electromotive forces in a loop equals

the sum of potential drops in the loop.

OR

The net voltage change in a closed loop is equal to zero.

Kirchoff’s second law is also known as the Kirchoff’s voltage law (KVL), it is the direct result of law of conservation of energy.

When a charge moves from a lower potential to a higher potential it gains energy but when it moves from higher potential to lower potential it looses energy. OR we can say that it gains energy when it moves through a source of e.m.f from its negative to positive terminal but it looses energy when there is a potential drop.

Hence if charge is moved round a closed path starting and ending at the same point (either clockwise or anticlockwise) then it will have the same final potential energy as its initial potential energy.

 While solving a complex network (a complex network is that electrical circuit which has two or more voltage sources), by Kirchoff’s voltage rule KVL,following steps must be taken:

- Direction of current flowing through all loops is assumed to be either clockwise or anticlockwise, or you can assume current in a loop in clockwise and in other loops anticlockwise.

- If the direction of that loop current is such that it flows through a voltage source from its negative to positive terminal, voltage change across that source of emf  is taken +E (E is the emf or voltage of the source).

- Similarly, if the direction of that loop current is such that it flows through a voltage source from its positive to negative terminal, voltage change across that source of emf is taken – E (E is the emf or voltage of the source).

- Kirchoff’s second rule KVL is then applied to each loop separately, finding the potential drop across each resistor in a loop by applying Ohm’s law (V = I R).

- Apply Kirchoff’s second rule KVL to each loop separately, assuming the current in that particular loop to be greater than current in its adjacent loop/s. You will get an equation for each loop. Solve these equations simultaneously. Thus, you will get the value of current in each loop.

- If current in a loop comes out to be negative, then its correct direction of flow will be opposite to its assumed direction, it means that if the direction of current assumed is clockwise and if it comes out to be negative then its correct direction of flow will be anticlockwise. With the help of each loop current determine the value of current in each resistor.

Wheatstone bridge:

Q.No.1: What is a Wheatstone bridge? Describe working of a Wheatstone

bridge.

Ans: If four resistances R1, R2, R3, and R4 are connected end to end in such a manner

I

that

I2

A

I1

they form a closed loop. A sensitive galvanometer through a key is connected across two opposite junctions whereas, a cell is connected across the remaining two junctions, then such an electrical circuit is known as Wheatstone bridge

R3

I

Wheatstone bridge

R1

I2

R4

I1

R2

C

B

I

I

G

D

+

–

When current is switched on in a Wheatstone bridge, some current “I” is delivered by the battery. It divides into two parts “I1” and “I2” at point “A”. These parts will further divide at points “B” and “D”. Some current will flow through the galvanometer, due to which it shows deflection. The magnitude and direction of current flowing through the galvanometer and the direction of deflection of its pointer depends upon the potentials at points “B”  and “D”. If point “B” is at a higher potential then current will flow from “B” to “D” and vice versa.

If points  “B” and “D” are at the same potential then no current will flow through the galvanometer, hence it’s deflection will be zero (null point is obtained). Under this condition Wheatstone bridge is said to be balanced.

Q.No.2: Derive a condition for a balanced Wheatstone bridge.

Ans:  For a balanced Wheatstone bridge potentials at points “B” and “D” must be equal

i.e. VB = VD or in other words, potential difference across “R1” must be equal to the potential difference across “R3”.

But	Potential difference across R1 = I1 R1

Potential difference across R3 = I 2 R3

			I 1 R1 = I 2 R3 …………… (i)

Similarly potential difference across R2 and R4 must be equal.

But	Potential difference across R2 = I1 R2

Potential difference across R4 = I2 R4

			I1 R2 = I2 R4 ……………..(ii)

Dividing equation (i) by equation (ii) we get,

I1 R1 =  I2 R3

I1 R2     I2 R4

			 R1 =   R3

R2      R4

This is the condition for a balanced Wheatstone bridge. If this equation is satisfied, the

bridge will be balanced i.e. no current will flow through its galvanometer and null point will be obtained. In other words, if R1 are R2 are in the same ratio as R3 and R4 then the bridge will be balanced i.e. no current will flow through the galvanometer.

- If three resistances are known in the above equation then the fourth one can be determined after balancing the bridge. Many electrical circuits used for measuring unknown resistance, such as meter bridge, post office box etc. are based on the principle of balanced Wheatstone bridge.

Thermoelectric current:

When junctions of two wires made of different materials are maintained at different temperatures an electromotive force emf is generated across their ends. This emf is called thermoelectric emf and the resulting current is called thermoelectric current.

Thermoelectric emf finds many applications in modern day life.

Devices such fire alarms, thermostats etc. work using thermoelectric emf.

Section – A Multiple-Choice Questions MCQs:

Q.No.1: Kirchhoff’s laws are useful in determining:

(A) current flowing in a circuit.

(B) emf and voltage drops in a circuit

(C) Power in a circuit.

(D) Only emf in a circuit.

Q.No.2:The resistance of a superconductor is	(A) finite.     (B) Infinite.

(C) Changes with every conductor.

(D) Zero.

Q.No.3: Reciprocal of resistance is called:

(A) Conductance.     (B) resistivity.

(C) Resonance.        (D) Capacitance.

Q.No.4: The graphical representation of Ohm’s law is:

(A) Parabola.     	(B) Hyperbola.

(C) Ellipse.         	(D) Straight line.

Q.No.5: A potential difference is applied across the ends of a wire. If the potential difference is doubled, the drift velocity of free electrons will:

(A) be quadrupled.     (B) be doubled.

(C) be halved. (D) remain unchanged.

Q.No.6: Internal resistance is the resistance offered by:

(A) Capacitor.        	(B) Resistor.

(C) Conductor.       (D) Source of emf.

Q.No.7: Power dissipation in a resistor can be calculated using which formula.

(A) P= V2/R		(B) P = I2R

(C) P = V I		(D) P= R/(V I)

Q.No.8: What is a potentiometer used for:

(A) Measuring electric current.

(B) Measuring electric charge.

(C) Measuring potential difference.

(D) Measuring electric resistance.

Q.No.9: A heat sensitive device whose resistivity changes with the change in temperature is called:

(A) Conductor.       	(B) Resistor.

(C) Thermistor.       	(D) Thermometer.

Q.No.10: A wire of uniform area of cross-section A length L and resistance R is cut into two parts. The resistivity of each part is:

(A) becomes zero.     (B) is halved.

(C) is doubled.      (D) remains same.

Q.No.11: Temperature coefficient of resistance is of a semiconductor is:

(A)Positive.   (B)Negative.(C)Zero.

(D)Can be positive or negative.

Q.No.12: The commercial unit of electrical energy is:    (2005,10 Karachi Board)

(A)ohm.  (B)watt.   (C)kwh   (D)ampere

Q.No.13: The rate of transfer of charges through a circuit is called:

(2006 Karachi Board)

(A)resistance.	(B)current.

(C)potential difference.(D)all of them

Q.No.14: When a resistor carries a current “I”, the power dissipation by it is “P”. If the same resistor carries the current of ‘3 I’, the power dissipation will be:					    (2007 Karachi Board)

(A)P  (B) P/3  (C)3P  (D) None of the above

Q.No.15: If a wire of uniform area of cross section is cut into two equal parts, the resistivity of each will be:

(2007 Karachi Board)

(A) Halved.		(B) Doubled.

(C) Remains the same.  (D) none of the above

Q.No.16: A piece of wire of length ‘L’ and an area of cross section ‘A’ has a resistance ‘R’. Another piece of wire of the same material and the same length but twice the area of cross section is connected end-to-end with the previous wire. The effective resistance is:   (2008 Karachi Board)

(A) R	  (B) 2    (C)  R    (D) ½ R

Q.No.17: In a house circuit all the electrical appliances are connected in parallel with the phase and the neutral to get:

(2008 KARACHI Board)

(A) Same current and potential difference.

(B)Different currents but same potential difference

(C) Different currents and different potential difference

(D) Same currents and same potential difference

Q.No.18: The electrical energy dissipated as heat in a resistor is:

(2016,10,9,6,5 Karachi Board)

(A) V2 R.  (B) V2 R t.   (C) I2 R t.  (D) I 2 R.

Q.No.19: Resistances of 10, 30 and 40 are connected in series, if the current in 10  is 0.1 A then the current through 40  will be:          (2019, 10 Karachi Board)

(A) 0.4A   (B) 0.3A   (C) 0.1A   (D)0.08A

Q.No.20: Resistivity of a material is , If area of cross section of the material is doubled and length is halved then the resistivity of the conductor will:

(2022, 18 Karachi Board)

(A)Remains constant   (B)Increase 4 times

(C) Decrease 2 times    (D) is one fourth

Q.No.21: Four bulbs of 10, 20, 30 and 40 are connected in series, the bulb that will shine the most is:

(2022 Karachi Board)

(A) 10     (B) 20   (C) 30   (D) 40 

Q.No.22: A current of 9 amp. flows in a circuit of three resistors connected in parallel. If each resistor has magnitude 5 , then current in the 3rd resistor is:

(2022 Karachi Board)

(A)9 amp  (B) 3 amp (C)1.5 amp. (D)0.5 amp.

Q.No.23: A 12 volt battery with internal resistance 0.1 is to be charged at 10 ampere. the charger must be of:

(2022 Karachi Board)

(A)11 volts (B)9 volts (C)13 volts (D)12 volts

Q.No.24: A 2.2 kW electric iron operates at 220 volt, current it draws:

(2022 Karachi Board)

(A)20 amp  (B)22 amp(C)10 amp  (D)5 amp.

Q.No.25: Total potential difference across the combination of three cells becomes maximum when:   (2003 Karachi Board)

(A) All the three cells are combined in series.

(B) All the three cells are combined in parallel.

(C) Two cells are connected is parallel and the third cell in series with the combination.

(D) Two cells are connected in series and the third cell in parallel with the combination.

Q.No.26: If 1A current flows through 2 m long wire, the charge flows through it in

2 hours will be:    (2023 Karachi Board)

(A) 3600 C  (B) 7200 C  (C) 1 C  (D) 600 C

Q.No.27: Reciprocal of resistance is called:

(2025, 26 Karachi Board)

(A) Resistivity.	(B) Conductance.

(C) Resonance.	(D) Capacitance.

Answers:

(1) (A) current flowing in a circuit.

(2) (D) Zero.

(3) (A) Conductance.

(4) (D) Straight line.

(5) (C) be halved.

(6) (D) Source of emf.

(7) (A),(B)If “V” is constant then

P= V2/R, If “I” is constant then P = I2R.

(8) (C) Measuring potential difference.

(9) (C) Thermistor.

(10) (D) remains same.

(11) (B)Negative.

(12) (C)kwh.

(13) (B)current.

(14) (C)3P

(15) (C) Remains the same.

(16) (C)  R

(17) (B)Different currents but same

potential difference.

(18) (C) I2 R t.

(19) (C) 0.1A.

(20) (A) Remains constant.

(21) (A) 10 .

(22) (B) 3 amp.

(23) (C)13 volts.

(24) (C)10 amp.

(25) (A) All the three cells are

combined in series.

(26) (B) 7200 C

(27) (D) Capacitance.

Numericals:

Q.No.1: The storage battery of a car has an emf of 12 V. If the internal resistance of the battery is 0.5 W, What is the maximum current that can be drawn from the battery?

Data:

EMF E = 12 V  Internal resistance r = 0.5 W

Max. current   =?

Solution:

Current drown from a battery is given by:

I =     E     .

(R + r)

Where R is the load resistance.

Maximum current can be drawn from a battery when load resistance is negligible.

I =     12            I = 24 amperes.

(0+ 0.5)

Q.No.2: A negligibly small current is passed through a wire of length 12 m and uniform cross section 4.010–7 m2 and its resistance is measured to be 6.0 W.

What is resistivity of the material at temperature of the experiment?

Data:

Length L = 12 m    Resistance R = 6.0 W

Area of cross section A = 4.010–7 m2

Resistivity 	r =?

Solution:

Resistance of a conductor of given material is given by:

R = r  L           r =  R A

A	             L

r =  6  4.010–7       r = 2.010–7 W - m

12

Q.No.3: In a potentiometer arrangement, a cell of emf 1.20 V gives a balance point at 40.0 cm length of the wire. If the cell is replaced by another cell and the balance point shifts to 74.0 cm, what is the emf of the second cell?

Data:

E1 = 1.2 V	L1 = 40.0 cm.

E2 =?		L2 = 74.0 cm.

Solution:

E1 = L1

E2    L2

E2 = E1 L2= 1.2  74

L1              40

E2 = 2.22 volt.

EMF of the second cell is 2.22 volt.

Q.No.4: (a) Three resistors 1W, 2W and 3W are combined in series. What is the total resistance of the combination?

(b) If the combination is connected to a battery of emf 24 V and negligible internal resistance, obtain the potential drop across each resistor.

Data:

R1 = 1 W         R2 = 2 W         R3 = 3 W    R = ?

(a) V = 24 volt    V1 = ?              V2 = ?       V3 = ?

Solution:

R = R1 + R2 + R3  (For series combination)

R = 1 + 2+ 3 = 6 W

Total resistance of the combination is 6 W

V = I R     (Ohm’s law)

I =  V  = 24  = 4 amp.

R      6

Hence 4 ampere current flows through the combination. Since in series combination same current flows through each resistance, therefore

V1 = I R1 = 4  1		V1 = 4 volt.

V2 = I R2 = 4  2	         V2 = 8 volts.

V3 = I R3 = 4  3	        V3 = 12 volts.

(To check the result, In series combination total voltage V = V1+V2+V3)

Q.No.5: From the given circuit find the value of “I”.		0.4 A

0.2A			0.6 A

0.5 A

I		0.7 A

Solution:

According to Kirchoff’s first law (KCL)

Total current flowing towards a junction =

Total current flowing away from the junction.

Since 0.2 amp., 0.7 amp. and 0.6 amp. currents are flowing towards the junction; therefore, their sum must be equal to the sum of currents “I”, 0.5 amp. and 0.4 amp. Which are flowing away from the junction. Hence,

S Itowards = S Iaway

0.2 + 0.7 + 0.6 = I + 0.5 + 0.4

1.5 = I + 0.9    I = 0.6 ampere.

Q.No.6: In a meter bridge with a standard resistance of 15 W in the right gap, the ratio of balancing length is 5:3, Find the value of  other resistance.

Solution:

Working of a meter bridge is based on the principle of balanced Wheatstone bridge.

For a balanced meter bridge

Ratio of resistances = ratio of balancing length Since the balancing lengths are in the ratio of 5:3, therefore, resistances will also be in the ratio of 5:3.

The other resistance will be 5 times the standard resistance.

Other resistance = 5  standard resistance

Standard resistance = 15 W

   Other resistance = 5  15

Other resistance = 75 W

Q.No.7: By KVL (Kirchoff’s Voltage Law), find current flowing through 10 W resistance.

D

C

R1=5W      R3=10W    R5=12W

R4= 8 W

R2= 15 W

E1=4V       I1	 	   I2                   I3

E2= 6 V

Data:

R1=5W   R2=15W  R3=10W    R4=8W

R5 =12W	‎‎‎E1 = 4 V	E2 = 6 V‎‎

‎Current flowing through 10W   I2 =?

Solution:

In this problem direction of currents I1, I2 and I3 in first, second and third loops is given as clockwise.

- Applying Kirchoff’s voltage law

(KVL) to first loop (assuming I1 > I2) we get:

E1 – I1 R1 – (I1 – I2) R2 = 0

4 – 5 I1 – (I1 – I2)15 = 0

4 – 5 I1 – 15 I1 + 15 I2 =0

4 – 20 I1 + 15 I2 =0   ………. (1)

- Applying Kirchoff’s voltage law

(KVL) to second loop (assuming I2> I3) we get:

– (I2 – I1)R2 – I2 R3 – (I2 – I3)R4 = 0

– (I2 – I1)15 – 10 I2 – (I2 – I3)8 = 0

–15 I2 + 15 I1 – 10 I2 – 8 I2 + 8 I3 =0

–33 I2 + 15 I1 + 8 I3 = 0   ………. (2)

- Applying Kirchoff’s voltage law

(KVL) to third loop (assuming I3 > I2) we get:

– (I3 – I2) R4 – I3 R5 – E2 = 0

– (I3 – I2) 8 – 12 I3 – 6 = 0

–8 I3 + 8 I2 – 12 I3 – 6 = 0

–20 I3 + 8 I2 – 6 = 0

–10 I3 + 4 I2 – 3 = 0   ………. (3)

To solve these equations simultaneously  from eq. (1)  we have:

4 – 20 I1 + 15 I2 = 0   ………. (1)

I1 = – 15 I2 – 4 = 15 I2 + 4

– 20		20

Put this value of I1 in eq. (2),

–33 I2 + 3 15 (15 I2 + 4) + 8 I3 = 0

20  4

–132 I2 + 45 I2 + 12 + 32 I3 = 0

– 87 I2 + 32 I3 + 12 = 0 ……… (A)

Now multiply eq. (3) by 32 and eq. (A) by 10 and then add them.

(–10 I3 + 4 I2 – 3      = 0) 32

(– 87 I2 + 32 I3 + 12 = 0) 10

128 I2 – 96 – 870 I2 + 120 = 0

– 742 I2 – 24 = 0

I2 =   24  =      I2 =  – 0.03235 Amp.

– 742

Negative sign shows that I2 is flowing in anticlockwise direction.

Current flowing through 10 W ( represented by R3) is 0.03235 Amp. In anticlockwise direction.

To check tour answer:

Find the value of current in

each resistor, put the value of I2 in eq. (1) :

4 – 20 I1 + 15 I2 =0   ………. (1)

I1 = – 15 I2 – 4 = 15 I2 + 4

- 20                20

I1 = 15 (– 0.03235)+ 4 =  3.51475

20                              20

I1 = 0.1757 amp.

Put the value of I2 in eq. (3) :

–10 I3 + 4 I2 – 3 = 0   ………. (3)

–10 I3 + 4 (– 0.03235) – 3 = 0

–10 I3 – 0.1294 – 3 = 0

I3  = 3.1294       I3 = – 0.31294 amp.

–10

Negative sign shows that I3 is anticlockwise.

 Current through R2:

(I1– I2)= 0.1757– – 0.03235 = 0.20805 amp

 Current through R4:

(I2–I3) =–0.03235 – –0.31294= 0.28059 amp

To verify your results:

Apply Kirchoff’s current law to junction C:

0.1757+ 0.03235 = 0.20805

0.20805 = 0.20805

Current flowing towards C= Current flowing

away from C

Apply Kirchoff’s current law to junction D:

0.31294 = 0.03235 + 0.28059

0.31294 = 0.31294

Current flowing towards D= Current flowing

away from D

Since current flowing in each branch satisfies Kirchoff’s laws, hence verified.

Q.No.8: A silver wire 2 m long is to have a resistance of 0.5 .

What should its diameter be?

Data:

Length of the wire 	           L = 2 m.

Resistance of the wire           R = 0.5 .

Resistivity of silver       = 1.52 10−8 -m

Diameter of the wire	            d =?

Solution:

R =   L .

A

0.5 = 1.52 10−8 2 .

A

A = 1.52 10−8   2  .                           A = 6.08 10−8 m2

0.5

But		A =  r2        (For circular wire)

6.08 10−8 = 22   r2

7

		r2 = 6.08 10−8

3.142

r2 = 1.935 10−8     .

	 r =  1.935 10−8

		r = 1.39 10−4 m

But diameter of the wire is given by:

		d = 2 r = 2  1.39 10−4

d = 2.78 10−4 m

Diameter of the wire will be 2.78 10−4 m.

Q.No.9: A 40  resistor is to be wound from platinum wire 0.1 mm in diameter.

How much wire is needed?

Data:

Resistance of the wire  R = 40

Diameter of the wire d = 0.1 mm

=   0.1   m = 110–4 m

1000

Resistivity of platinum      =1110–8 -m

Length of wire		      L =?

Solution:

Diameter of the wire d = 2r

	 r =  d  = 110–4 m = 510–5 m

2	       2

Area of cross section of the wire:

A =  r2 = 22 (510–5)2

7

A = 7.854 10–9m2

Resistance of a wire is given by:

R =   L  .

A

40 = 1110–8        L        .

7.85410–9

	 L = 40 7.85410–9.    L = 2.86 m

1110–8

2.86 m long wire will be needed.

Q.No.10: A resistor is made by using a 50 meter nichrome wire of diameter 0.8 mm at 0C. Calculate its resistance at 50C.

(Given: resistivity  = 1.110–6 -m

and           = 0.0002 C–1 at 0C.)

(2009 Karachi Board)

Data:

Length of the wire 	L = 50 m

Diameter of wire   d = 0.8mm = 0.810–3 m

          r =  d   = 0.8 10–3 m = 0.410–3 m

2           2

Resistivity of nichrome  = 1.110–6 -m

Temperature coefficient

 = 0.0002 C–1 at 0C

Resistance at 50C ,    R50 =?

Solution:

Resistance of the wire at 0C is given by:	R0 =    L  .

A

	R0 = 1.110–650

r2

	R0 = 1.110–650 =109.42 

 (0.410–3)2

	 Resistance at 50C is given by:

R50 = R0 (1+ t)

R50 = 109.42 (1+ 0.0002 50)

	R50 = 110.52 

Q.No.11: A 50 ohm resistor is to be wound from a platinum wire 0.1 mm in diameter.

How much wire is needed?

(Resistivity of the wire =1110–8.m)

(2015,06,03,1999,85 Karachi Board)

Data:

Resistance		        R = 50 

Diameter     d = 0.1 mm = 0.110–3 m

Resistivity   	         =1110–8 .m

Length of wire	needed         L = ?

Solution:

Radius of the wire:

r =  d  = 0.110–3 = 0.0510–3 m

2	       2

Area of cross section of the wire will be:

A = r2 = 3.143 (0.0510−3)2

= 7.85410−9 m2

	R =   L 	  L =  RA .

A		  

	L =  50.85410−9    L = 3.57 m

1110−8

3.57 m long wire will be needed.

Q.No.12: A 20 m length of cable has a cross-sectional area of 1 mm2. and a resistance of 5 ohm. Calculate the resistivity of the cable.

(2025 Karachi Board)

Data:

Length of cable	        L = 20 m

Area of cross-section A=1 mm2= 110–6 m2.

Resistance		        R = 5 

Resistivity	   	         = ?

Solution:

Resistance of a conductor is given by:

R = r  L

A

	r = R A = 5  110–6 = 2.510–7 W-m

L	          20

 Resistivity of the cable is 2.510–7 W-m.