According to new syllabus of all

intermediate boards of Sindh.

Prof. Rawala’s ……..

Physics

Helping book

For

Class XI

(All science students)

Unit -6

Fluid statics

Notes ……………………………………………………… … (03)

Multiple choice questions M.C.Qs.	………….…......................   . (08)

Numericals from text book and Karachi board papers……………..…..(10)

Contents

- Fluids, fluid statics, Pascall’s law, applications of Pascall’s law.

- Hydraulic brake system, hydraulic lift.

- Archimede’s principle, principle of floatation.

- Surface tension.

Thoroughly revised and

updated

2026 – 2027



Detailed notes.

Multiple Choice Questions (M.C.Q’s).

Solution of text book numericals.

MCQs and solution of numericals from

Karachi board past papers.

Prof.(r) Mohammed Hussain Rawala.

B.Sc. (Hons.), M.Sc.(Physics)

Former head of physics department

Saint Patrick’s college, Karachi.

Unit. 6….. 

Fluid statics

Fluids:

Q.No.1: What is a fluid?

Ans: There are three states of matter. States of matter that can flow are called fluids.

Since liquids and gasses can flow therefore, broadly speaking they are called fluids.

Q.No.2: What is a fluid statics?

Ans: Fluid statics is a branch of physics in which we study properties of a stationary fluid (i.e. a fluid at rest).

Similarities and differences between

physical properties of liquids and gasses:

| Physical property | Gasses | Liquids |
| --- | --- | --- |
| Volume. | No fixed volume, assumes volume of the container. | Fixed volume (although there is a slight change in volume when high external pressure is applied). |
| Shape. | No fixed shape. | No fixed shape. |
| Density. | Low density. | Higer density compared to gasses. |
| Compressibility | Highly compressible. | Low compressibility. |
| Viscosity. | Negligibly low viscosity. | Higher viscosity, some liquids are highly viscose. |

Q.No.3: What is an ideal fluid?

Ans: An ideal fluid is that which is incompressible, it means that its volume cannot be changed by applying external pressure on its surface. Since density is mass per unit volume, therefore, density of an ideal fluid remains constant since its mass and volume both are not changing. In other words, gasses are not an ideal fluid, volume of liquids do not change appreciably on applying external pressure on their free surface, therefore, they can be considered as ideal fluids in ordinary sense.

Pascal’s law:

Q.No.4: State Pascal’s law.

Ans:	According to Pascal’s law:

“When pressure is changed at any point in a stationary fluid,

this change in pressure is equally transmitted to all points in the fluid”.

Applications of Pascal’s law:

Q.No.5: Describe some applications of Pascal’s law.

Ans:	(1) Hydraulic brake system:

Hydraulic brake system of cars is an application of Pascal’s law. Brake system of a car has a cylinder, called master cylinder, it is attached to the brake padel. When the driver presses the brake padel this increased pressure pushes the piston in the master cylinder. This increased pressure is transmitted equally to each brake cylinder provided with rear wheels in a front wheel drive car. Increased pressure is transmitted to entire braking system through the incompressible brake fluid (brake oil) filled in each cylinder and a strong metallic tube that connects all the cylinders with each other. In modern cars front wheels are provided with strong metallic disc rotating along with the wheel.

Increased transmitted pressure opens the brake pad which applies a strong force of friction to the rotating wheel drum, forcing the car to slow down. As soon as the brake padel is released pistons in all cylinders move back to their original positions, a spring pulls back the brake shoe and the car is again able to move as before.

(2) Hydraulic lift:

Pascal’s law is applied to lift heavy objects such as a heavy car by applying a small force as in hydraulic lift.

Hydraulic lift has a U-shaped container filled with an incompressible liquid (a liquid that cannot be compressed on applying force on it, so that its volume does not change keeping it density constant. Real liquids show slight change in their volume on being compressed; hence this is true for an ideal liquid). Container has two cylinders at its two ends; both provided with pistons. One of the cylinders is narrower than the other cylinder which carries a platform on which a car can be parked easily.

F1

F2

Hydraulic lift

If a force F1 is applied on the piston of smaller cylinder of area A1, then pressure applied will be P1= F1/A1. Since the liquid is incompressible, therefore, according to Pascal’s law, this increased pressure will be equally transmitted throughout the liquid. Pressure on piston of wider cylinder will be same as the pressure on smaller piston, but since this piston is of large area “A2” therefore, force on this piston will also be large so that pressure on this piston is the same. If F2 is the output force at piston of wider cylinder of area A2 the pressure on it will be P2 = F2/A2.

But     input pressure P1 = Out put pressure P2

Therefore,	                                  F1  = F2 .

A1     A2

	              F2 =   A2

F1      A\1

F2 =  A2  F1

A1

We can see from this formula that the value of output force (Force required to lift heavy load i.e. a car) can be increased to any desired value by increasing the area of output (larger) piston that carries the car.

Mechanical advantage of hydraulic lift: Mechanical advantage of any machine is the ratio of output force to input force (Load lifted to effort applied). It tells you about how many times the force required to lift a load will be greater than the applied force.

In this case:

Mechanical advantage of a Hydraulic lift = F2

F1

If mechanical advantage of a lift is 100 then that machine will be able to lift a 100 times heavier load, it means that this particular machine will be able to lift a 10000 Newton car by applying only 100 Newton on the smaller piston.

Example: Let the radius of smaller piston be 25 cm. (= 0.25 m), radius of wider piston be 1 m. and the applied force is 90 N, then:

F2 =  A2  F1 =  p r22 F1

A1	            p r12

F2 =   r22 F1 = (1)2  90 = 16  90 = 1440 N

r12	 (0.25)2

From this example you can see that output force can be increased by 16 times i.e. only

90 N input force will be needed to lift 1440 N load (this is for an ideal lift, free of friction, if there is friction then a part of input energy will be wasted to overcome friction).

Mechanical advantage of this lift will be:      Output force e =  F2= 1440 = 16

input forc            F1     90

Upthrust (or Buoyant force):

Q.No.6: What is upthrust (or Buoyant force)?

Ans:	When a body is immersed in a fluid (any liquid or a gas) it experiences an upward force, exerted by the fluid, this force is known as upthrust or buoyant force.

Pressure in a fluid depends mainly upon its density and depth, as a body is immersed deeper into a fluid there will be a difference in pressure of the liquid between lower and upper surfaces of the body.

We know that pressure in a fluid is equal at all points and acts in all directions lying at the same depth, hence when a body is fully immersed into a fluid then pressure exerted by the fluid on the upper surface of the body is less than the pressure at its lower surface (Lower surface is deeper)

This difference in pressure results in an upward directed force acting on lower side of the body, called upthrust or buoyant force.

Q.No.7:Why does a body appear to be lighter when immersed in a fluid?

Ans:	Due to upthrust body appears to be lighter when immersed in a fluid i.e. its apparent weight is less than its true weight.

Upthrust helps io support a body and tends to prevent it from sinking.

Since density of liquids is more than the density of gasses due to which liquids exert more pressure, in other words, upthrust in liquids is much more than in gasses.

- If density of the object is less than the density of liquid, the object will float

- If density of the object is equal to the density of liquid, the object will remain

suspended in the liquid just like a fish.

- If density of the object is more than the density of the liquid the object will sink,

because the upthrust will be smaller than weight of the body and will not be enough to support it.

Archimede’s principle:

Q.No.8:State Archimede’s principle.

Ans:	According to Archimede’s principle:

“When an object is immersed into a liquid it experiences

upthrust equal to weight of the liquid displaced.”

The magnitude of upthrust depends upon:

- Volume of the body immersed, because more is the volume of the body immersed more is the volume of fluid displaced, more will be the upthrust.

- Density of the fluid more is the density of fluid more will be the upthrust.

Volume of liquid displaced = volume of body immersed. If a body is fully immersed then the volume of liquid displaced is equal to total volume of the body but if a body is partially immersed then volume of liquid displaced will be equal to volume of the body inside the liquid.

Due to upthrust, exerted by the liquid the object appears to loss some of its weight. Hence a body immersed in a liquid appears to be lighter. In other words:

Apparent weight of a body = its actual weight – upthrust

Density r = mass m / volume V

mass of liquid displaced m = density of liquid r

 volume V of liquid displaced.            m = r  V

Weight of liquid displaced (Buoyant force)= m g

Weight of liquid displaced (Buoyant force)= r V g

The magnitude of upthrust or buoyant force, according to Archimede’s principle, is given by:

Buoyant force = density of fluid  Volume of fluid

displaced  acceleration due to gravity.

Buoyant force = r V g

Principle of floatation:

A needle made of steel sinks in water whereas huge cargo ships made of steel do not sink, instead they float. The reason for this observation is that a needle displaces a small volume of liquid and the weight of liquid displaced is quite small as compared to weight of the needle. Hence the upthrust acting on the needle is not enough to support it, therefore, it sinks. A huge cargo ship because of its shape and because of large empty spaces inside the ship displaces large volume of liquid. Weight of this large volume of liquid displaced is enough to support a loaded cargo ship and prevents it from sinking.

In other words:

“ A floating object displaces weight of fluid equal to its own weight”.

Ships and submarines are designed on this principle; principle of floatation is based on Archimede’s principle.

In gasses (eg air):

“An object (e.g. a balloon) surrounded by air experiences

a force equal to weight of air displaced”.

Airships and hot air balloons etc. displace large quantity of air, such that if weight of air displaced is more than balloon’s own weight than balloon rises up (because there will be more upthrust). But if weight of air displaced is less than balloon’s own weight then balloon descends (because there will be less upthrust). If weight of air displaced is equal to balloon’s own weight than balloon hovers at a constant altitude. (because there will be equal upthrust).

Surface tension:

Q.No.9:What is surface tension?

Ans:	Surface tension of a liquid may be defined as:

“The force per unit length acting on either side of an imaginary line drawn

on the liquid surface at rest. Direction of this force is along

tangent to the surface and is perpendicular to the line”

Surface tension =  F        (SI unit N/m)

L

Due to surface tension, surface of a liquid acts as a membrane. It is the surface tension due to which surface of a liquid tends to have minimum surface area. Due to surface tension, a liquid supports a needle, liquid surface assumes a concave shape, liquid rises in a capillary tube and a liquid bubble is spherical in shape and so on.

Section – A Multiple-Choice Questions MCQs:

Q.No.1: A completely submerged object always displaces its own ………

(A) weight of fluid.         (B) volume of a fluid

(C) density of a fluid.     (D) Area of a fluid.

Q.No.2: The pressure exerted on the ground by a man is greater when:

(A) he stands with both feet flat on the ground

(B) he stands flat on one foot.

(C) he stands on the toes of one foot.

(D) he lies down on the ground.

Q.No.3: In a stationary homogeneous liquid:

(A) pressure is the same at all points.

(B) pressure depends on the direction.

(C) pressure is independent of any atmospheric

pressure on the upper surface of the liquid.

(D) pressure is the same at all points at

the same level.

Q.No.4: one piston in a hydraulic lift has an area that is twice the area of all others. When the pressure at the smaller piston is increased by Dp the pressure at the larger piston:

(A) increases by 2 Dp   (B) increases by Dp/2

(C) increases by Dp.   (D) increases by 4 Dp

Q.No.5: In vacuum, an object has:

(A) No buoyant force.     (B) no mass.

(C) no weight.       	       (D) none of these.

Q.No.6: The pressure at the bottom of a pond does NOT depend on:

(A) Water density.     (B) depth of the pond.

(C) surface area of the pond (D) None of these

Q.No.7: A rock suspended by a weighing scale weighs 5 N out of water and 3 N when submerged in water. What is the buoyant force on the rock?

(A) 2 N.      (B) 5 N.      (C) 8 N.     (D) 15 N.

Q.No.8: “An object completely submerged in a fluid displaces its own volume of fluid”. This is:

(A) Pascal’s paradox. (B) Archimede’s principle

(C) Pascal’s principle.(D)true, but none of the above

Q.No.9: Salt water has greater density than freshwater. A boat floats in both freshwater and salt water. The buoyant force on the boat in saltwater is …………. that in freshwater.

(A) equal to     	(B) smaller than

(C) larger than	(D) same as

Q.No.10: You fill a tall glass with ice and then add water to level the glass’s rim, so some fraction of the ice floats above rim. When ice melts, what happens to water level?

(A) water overflows the rim.

(B) water level drops.

(C) water level stays at the rim.

(D) it depends on the difference in

density between water and ice.

Q.No.11: The S.I unit of surface tension is:

(2025 Karachi Board)

(A) N.	    (B)  N       (C)  N	    (D)  m  .

m             m3	           N

Q.No.12: The buoyant force on an object in salt water compared to that in fresh water will be:	       (2026 Karachi Board)

(A) the same.	(B) smaller.

(C) Zero.		(D) Greater.

Answers:

(1) (A) weight of fluid.

(2) (C) he stands on the toes of one foot.

(3) (D) pressure is the same at all points at the same level.

(4) (C) increases by Dp.

(5) (A) No buoyant force.

(6) (C) surface area of the pond.

(7) (A) 2 N.

(8) (N) Archimede’s principle

(9) (C) Larger than

(10) (C) Water level stays at the rim.

(11) (B) N .

m

(12) (D) Greater.

Numericals:

Q.No.1: In a hydraulic press a force of 20 N is applied to a piston of area 0.20 m2. The area of the other piston is 2.0 m2. What is:

(a) Pressure transmitted through the fluid

(b) The force on the piston.

Data:

Force on smaller piston  	F1= 20 N

Area of smaller piston        	A1=0.20 m2

Areas of larger pistons    	A2 = 2.0 m2

- Pressure transmitted             P =?

- Force on the piston (Larger)  F2 =?

Solution:

(a) Pressure is equally transmitted everywhere (Pascal’s law), Hence pressure on smaller piston:

P = F1  =   20   = 100 N/m2 or 100 Pa.

A1     0.20

(b) Force on the larger piston is given by:

	F2 = P A2 = 100  2.0              F2 = 200 N.

Q.No.2: The pressure in a water pipe on the ground floor of a building is 4105 Pa but three floors up it is only 2105 Pa. What is height between the ground floor and the third floor? Water in the pipe may be assumed to be stationary, density of water =1103 kg/m3.

Data:

Pressure of water at ground floor P1 = 4105 Pa

Pressure of water at third floor      P2 = 2105 Pa

Density of water                        r = 1103 kg/m3.

Height between the floors               h = ?

Solution:

Difference of pressures = P1 – P2

But 		   P1 – P2 = r g h

	               4105 – 2105 = 1103  9.8 h

2105 = 9.8 103 h

	h =  2  105                  h = 20.4 m

9.8 103

Q.No.3: The small piston of hydraulic press has an area of 10.0 cm2. If the applied force is 50.0 N, what must the area of the large piston be to exert a pressing force of 4800 N ?

Data:

Area of small piston A1=10.0 cm2= 0.001m2

Applied force             F1 = 50.0 N

Output force              F2 = 4800 N

Area of large piston  A2 =?

Solution:

According to Pascale’s law:

P1 = P2		OR         F1/A1 = F2/ A2

- A2 = F2 A1 = = 4800  0.001

F1	       50.0

A2 = 0.096 m3 = 960 cm3

Q.No.4: Mechanical advantage of a hydraulic jack is 420. Find weight of the heaviest automobile that can be lifted by an applied force of 55 N.

Data:

Mechanical advantage    M.A = 420

Applied force                       F = 55 N.

Heaviest weight lifted         W =?

Solution:

By definition:  M.A =  W  = Output force (Load lifted)

F     Input force (applied force)

W = M.A  Input or applied force F

W = 420  55 		W = 23100 N.

Q.No.5: A flat-bottom river barge is 30 ft wide, 85 ft long and 15 ft deep.

(a) how many ft3 of water will displace while the top stays 1 m. above the water.

(b) What load in tons will the barge contain under these conditions if the empty barge weighs 160 tons in dry dock.

Data:

Length L = 85 ft.    Width (breadth) b = 30 ft.

Depth   h = 15 ft

(a) Volume of water displaced V=?

(b) Load in the barge                  = ?

Weight of barge            W =160 tons.

Solution:

(a) Volume displaced V= L b (depth immersed)

Volume displaced V = 85  30  (15–3.28)

(1m above water =3.28ft)      = 29886 ft3.

But 1 m3 = (3.28)3 ft3.

 Volume displaced V = 29886 m3= 847 m3.

(3.28)3

(b) Weight of water displaced =Bouncy = r V g

= 1  103  847  9.8 = 8300.6 N

(Density of water is taken as r = 1  103 kg/m3)

But 1 tone = 9807 N

       Bouncy = 8300.6 tones = 846.4 tones.

9807

Load in the barge=Bouncy – weight of the barge

Load in the barge = 846.4 –160 = 686.4 tones

Q.No.6: A canal lock gate is 20 m wide and

10 m deep. Calculate the thrust acting on it assuming that the water in the canal is in level with the top of the gate. Density of water is 1000 kg/m3.

Data:

Width W = 20 m,	Depth h = 10 m

Density of water r = 1000 kg/m3.

Thrust                 F = ?

Solution:

Average pressure on the gate P=r g (average depth)

= 1000  9.8  10 = 49000 N/m2.

2

Thrust F = P A = 49000 (20  10)

Thrust F = 9800000 N = 9.8 106 N.

Q.No.7: A tank 4 m long, 3 m wide and 2 m deep is filled to the brim with paraffin (density 800 kg/m3). Calculate the pressure on the base.

What is the thrust on the base?

Data:

Length L = 4 m.  width (or breadth)W = 3 m

Depth   h = 2 m        density r = 800 kg/m3

Pressure P = ?	Thrust -?

Solution:

Since P = Force =  mg

Area       L b

Density r= mass   =    m         m = r L b h

Volume     L b h

	P = r L b h g = r g h

L b

Pressure on the base P = 800  9.8  2

= 15680 N/m2.(or Pa)

Thrust on the base = Pressure  Area

Thrust = 15680  L b =1568043 = 188160 N.

Q.No.8: A rectangular boat is 4.0 m wide, 8.0 m long and 3.0 m deep.

(a) How much water will it displace if the top stays 1 m above the water?

(b) What load will the boat contain under these conditions if the empty boat weighs 8.60104 N in dry dock?

Data:

Length L = 8.0 m.	Width W = 4.0 m

Depth of boat inside water h = 3.0 – 1.0 = 2.0 m.

- Volume of water displaced V =?

- Load in the boat                 W =?

Solution:

(a) Water displaced = volume of boat submerged

= L W h = 8.04.02.0 = 64.0 m3.

(b) Bouncy = Weight of boat + Load in the boat

But   Bouncy = weight of liquid displaced = r g V

        r g V = Weight of boat + Load in the boat

	Load in the boat = r g V – Weight of boat

Load in the boat=10009.864 – 8.60104

Load in the boat = 627200 – 8.60104

	Load in the boat = 541200 N.

Q.No.9: A hot air ballon has a volume of

2200 m3. The density of air at temperature of 20oC is 1.205 kg/m3. The density of the hot air inside the ballon at a temperature of 100oC is 0.946 kg/m3.

How much weight can the hot air balloon lift?

Data:

Volume of balloon volume of displaced air V=2200 m3

Density of air inside the ballon     r1= 0.946 kg/m3.

Density of air outside the ballon   r2= 1.205 kg/m3.

Weight lifted 			W = ?

Solution:

Weight of balloon = r1 g V = 0.946  9.8  2200

= 20395.76 N

Upthrust = r2 g V = 1.205  9.8  2200

= 25979.8

But Upthrust = Weight of balloon + Weight lifted

Weight lifted = Upthrust – Weight of balloon

	Weight lifted = 25979.8 – 20395.76

	Weight lifted = 5584.04 N.

(This gives us maximum load that can be lifted)

Q.No.10: A spherical balloon has a radius of 7.15 m and is filled with helium. How large a cargo can it lift, assuming that the skin and structure of the balloon have a mass of

930 kg? Neglect the buoyant force on the cargo volume itself.

Data:

Radius of balloon r = 7.15 m

Volume of balloon V = 4  p r3 =  4    22  (7.15)3

3	      3	   7

Volume of balloon V = 1531.73 m3.

Mass of balloon    m = 930 kg

Cargo lifted           W = ?

Density of Helium r1= 0.179 kg/m3.

Density of air        r2 = 1.29 kg/m3.

Solution:

Upthrust = Weight of balloon + Weight of helium

+ Cargo lifted

 Cargo lifted = Upthrust – Weight of balloon

– Weight of helium

 Cargo lifted = r2 g V – m g – r1 g V

(volume of air displaced=volume of helium or of balloon)

 Cargo lifted = 1.299.81531.73 – 9309.8

– 0.1799.81531.73

 Cargo lifted =19364.13 – 9114 – 2686.96

 Cargo lifted =19364.13 – 11800.96

	Cargo that can be lifted = 7563.17 N.

Q.No.11: A tank 4 m long, 3 m wide and 2 m deep is filled to the brim with paraffin of density 800 kg/m3.

Calculate the pressure on the base.

(2025 Karachi Board)

Data:

Length L=4 m. width (or breadth)W= 3 m

Depth   h = 2 m     density r = 800 kg/m3

Pressure P = ?

Solution:

Since P = Force =  mg

Area       L b

Density r= mass   =    m         m = r L b h

Volume     L b h

	P = r L b h g = r g h

L b

Pressure on the base P = 800  9.8  2

= 15680 N/m2.(or Pa)

Q.No.12: The small piston of hydraulic press has an area of 10.0 cm2. If the applied force is 50.0 N, what must the area of the large piston be to exert a pressing force of 4800 N ?

(2026 Karachi Board)

Solution: See solution of Q.No.3 of this chapter.